String Grid Smoothing Macro
User Reference Guide

A practical guide for configuring, running, and reviewing String_grid smoothing macro.

Author: KLP | Published: 28 May 2026

Figure 1: Macro panel structure and parameter groups.

# 1. Purpose

The String Grid Smoothing macro is a 12d Model V15 macro for smoothing elevation values in an existing String_Grid element. It creates a new output String_Grid and is intended to leave the original input grid available for comparison.

The macro is useful for controlled reduction of local terrain noise, isolated elevation spikes, and rough contour behaviour in gridded terrain data. It can be used with LiDAR-derived DEMs and other gridded terrain surfaces, but the results must be checked before design, drainage, volumes, flood modelling, or construction use.

Warning: The macro changes the elevations written to the output String_Grid. It is not only a display or rendering effect.

# 2. Important Warnings

Save the 12d project before processing.

Use conservative settings first, then increase smoothing only if the result still contains unacceptable noise.

Inspect cross-sections through kerbs, drains, stream beds, road crowns, batters, retaining walls and other critical features.

Smoothing can fill narrow channels or soften sharp terrain breaks if settings are too aggressive.

The macro does not classify point clouds, validate survey accuracy, or identify drainage morphology.

Large grids may take several minutes. 12d Model may appear busy while the macro is running.

# 3. What the Macro Does

- Reads the selected input String_Grid geometry, range, spacing, and heights.

- Copies valid grid heights into memory.

- Applies the selected processing stages in sequence: Median, then Bilateral, then Gaussian.

- Creates a new output String_Grid and writes final heights to it.

- Prints a processing report to the 12d Output Window.

Note: Gaussian settings are not changed by the live preset selector in the current macro version.

# 4. Recommended Workflow

Select the input String_Grid.

Enter or confirm the String_grid resolution and vertical RMSE.

Choose a Light, Medium, or Strong smoothing preset.

Review the Median and Bilateral values that are automatically populated.

Manually adjust values where the site contains sensitive features.

Confirm the output String_grid name and output model.

Press Process and wait for the Output Window report.

Build or refresh TINs/contours from the original and smoothed grids.

Compare sections through critical terrain features before accepting the output.

# 5. Panel Overview

The panel is divided into Input / Output controls and three processing sections. Controls should be read from top to bottom.

| Control | Purpose | Notes |
| --- | --- | --- |
| Input String_grid | Selects the existing String_Grid to smooth. | Required. |
| String_grid resolution (m) | Cell spacing used by preset logic. | Default 1.000 m. Must be > 0. |
| String_grid vertical RMSE (m) | Vertical accuracy value used by preset logic. | Default 0.035 m. Must be > 0. |
| Smoothing preset | Light / Medium / Strong. Automatically updates Median and Bilateral fields. | Manual edits are overwritten if the preset, resolution, or RMSE is changed again. |
| Output String_grid name | Name of the new smoothed grid element. | Required. |
| Output model | Model where the new smoothed grid will be placed. | Required. |
| Process | Runs the smoothing process. | Does not run when only changing presets. |
| Finish | Closes the panel. |  |
| Help | Opens standard panel help/about function if configured. |  |

# 6. Live Preset Logic

The macro has live preset logic for Median Spike Correction and Bilateral Edge-Aware Smoothing. When the user changes String_grid resolution, String_grid vertical RMSE, or Smoothing preset, the Median and Bilateral parameter boxes are recalculated immediately.

Warning: Changing the preset, resolution, or RMSE after manually editing Median or Bilateral values will overwrite those manual edits.

| Preset | Intent | Typical use |
| --- | --- | --- |
| Light | Conservative smoothing and feature preservation. | Channels, stream beds, kerbs, table drains, road features, sharp batters. |
| Medium | General DEM cleanup with moderate smoothing. | General terrain, residential lots, broad ground surfaces. |
| Strong | More aggressive smoothing. | Broad open terrain only. Avoid near design-critical drainage or sharp features. |

| Preset | Med. window | Spike tol. | Med. max change | Min. neigh. | Bilat. window | Spatial sigma | Range sigma | Noise threshold | Bilat. max change |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Light | 3 | 4 × RMSE | 1.5 × RMSE | 5 | 3 | 1 × resolution | 1.5 × RMSE | 0 | 1 × RMSE |
| Medium | 3 | 5 × RMSE | 2 × RMSE | 6 | 5 | 2 × resolution | 2 × RMSE | 0.25 × RMSE | 1.5 × RMSE |
| Strong | 5 | 6 × RMSE | 3 × RMSE | 8 | 5 | 3 × resolution | 3 × RMSE | 0.5 × RMSE | 2 × RMSE |

Note: The supplied spreadsheet can be used as a companion decision-support tool, but the macro also applies this preset logic directly for Median and Bilateral settings.

# 7. Input / Output Parameters

| Widget | What it is | Expected values | Impact |
| --- | --- | --- | --- |
| Input String_grid | The source String_Grid element. | Existing valid String_Grid. | Wrong input gives wrong output; original grid is retained for comparison. |
| Output String_grid name | Name for the new smoothed String_Grid. | Any valid unique 12d element name. | Use a name that records the processing method or preset. |
| Output model | Model where the output grid is written. | Existing or new model. | Keep output separate from the source model if comparison is required. |
| String_grid resolution (m) | Grid cell size used by preset calculations. | Typically 0.25 to 2.0 m for DEM grids. | Higher values increase spatial sigma in the preset logic. |
| String_grid vertical RMSE (m) | Expected vertical random error used by preset calculations. | Use survey/LiDAR report value. Example: 0.035 m. | Higher RMSE increases tolerance, range sigma, and change caps. |
| Smoothing preset | Selects Light, Medium, or Strong calculated parameters. | Start with Light. | Affects Median and Bilateral fields only. |

# 8. Median Spike Correction

Median Spike Correction is intended for isolated high or low cells. It is not a general smoothing method. For each valid cell, the macro compares the cell elevation with the median of valid neighbouring cells. Replacement occurs only if the difference exceeds the spike tolerance, and the replacement is capped by Median maximum vertical change.

Warning: In a narrow channel, neighbouring cells may mostly be higher than the invert. Aggressive median settings can therefore lift or fill the channel bed.

| Widget | What it does | Expected range | Effect / risk |
| --- | --- | --- | --- |
| Apply median spike correction | Turns the median stage on or off. | ON/OFF. | Use ON for spike/noise cleanup; OFF for diagnostics or if channels are being altered. |
| Median window size | Square window size in grid cells. | Odd values. Usually 3; use 5 with caution. | Larger windows catch broader spikes but can damage drains, kerbs and narrow features. |
| Spike tolerance | Minimum difference from local median before correction occurs. | Approx. 3–6 × vertical RMSE. | Lower values alter more cells; higher values only remove larger spikes. |
| Median maximum vertical change | Caps the amount a cell can move when median replacement is triggered. | Approx. 1–3 × vertical RMSE. | Lower values protect features; higher values remove bigger spikes but may fill/cut real terrain. |
| Minimum valid neighbours | Minimum valid surrounding cells needed before the median test is applied. | 3–8 for a 3×3 window; higher for larger windows. | Higher values reduce gap-edge artefacts but may skip corrections near sparse data. |

## Median decision rule

If |Zcell − Zmedian| > spike tolerance, the cell moves toward Zmedian, limited by Median maximum vertical change. Otherwise the original cell value is retained.

# 9. Bilateral Edge-Aware Smoothing

Bilateral smoothing is the primary smoothing method for civil terrain in this macro. It uses both horizontal distance and elevation difference. Nearby cells with similar elevations have more influence; cells that are farther away or vertically different have less influence.

| Widget | What it does | Expected range | Effect / risk |
| --- | --- | --- | --- |
| Apply bilateral edge-aware smoothing | Turns the Bilateral stage on or off. | ON/OFF. | Recommended for controlled DEM smoothing where edge preservation matters. |
| Bilateral window size | Square neighbourhood size. | Usually 3 or 5. Use odd numbers. | Larger windows smooth more but increase risk to narrow channels and runtime. |
| Bilateral spatial sigma | Controls how weight decays with horizontal distance. | Approx. 1–3 × grid resolution. | Higher values allow farther cells to influence the result. |
| Bilateral range sigma | Controls how much vertical difference is treated as similar. | Approx. 1.5–3 × vertical RMSE. | Lower preserves edges; higher smooths more but can soften kerbs, drains and channel inverts. |
| Bilateral noise threshold | Ignores very small calculated changes. | 0 to 0.5 × vertical RMSE. | Higher values leave small roughness unchanged; lower values allow finer changes. |
| Bilateral maximum vertical change | Caps the per-cell Bilateral elevation movement. | Approx. 1–2 × vertical RMSE. | Lower values anchor the output to source data; higher values allow stronger smoothing. |

## Bilateral weighting

Weight = spatial weight × range weight

spatial weight = exp(-(distance²) / (2 × spatial sigma²))

range weight = exp(-((centre Z − neighbour Z)²) / (2 × range sigma²))

Note: Bilateral is edge-aware, but it does not understand drainage morphology. A stream-bed profile can still be lifted if most valid neighbours around the invert are higher.

# 10. Gaussian Broad Surface Smoothing

Gaussian Broad Surface Smoothing performs broad distance-weighted averaging of actual output grid elevations. It is not a display setting and it is not edge-aware.

Warning: Avoid Gaussian smoothing near kerbs, drains, channels, retaining walls, bridge edges, road crowns, batters, and flood modelling flow paths unless broad surface averaging is acceptable.

| Widget | What it does | Expected range | Effect / risk |
| --- | --- | --- | --- |
| Apply Gaussian broad smoothing | Turns the Gaussian stage on or off. | OFF by default. | Use only when broad averaging is acceptable. |
| Gaussian radius | Search radius around each cell, in model units. | Usually around 1–3 grid cells for light use. | Larger radius smooths broader areas and increases runtime. |
| Gaussian sigma | Controls smoothing strength within the radius. | Often 0.5–2.0 for 1 m grids. | Higher sigma gives a flatter/softer result. |
| Gaussian passes | Number of repeated Gaussian smoothing passes. | Usually 1. Use 2 only with caution. | More passes increase smoothing and runtime approximately linearly. |
| Gaussian maximum vertical change | Caps movement per Gaussian pass. | Conservative: 0.05–0.10 m for fine civil features; higher only for broad terrain. | Higher values can round sharp features. |

# 11. Parameter Effect Summary

| Parameter | If increased | If decreased | Main risk |
| --- | --- | --- | --- |
| Resolution | Preset spatial sigma increases. | Preset spatial sigma decreases. | Wrong resolution gives unsuitable Bilateral settings. |
| Vertical RMSE | Tolerances, sigmas and change caps increase. | Settings become more conservative. | Wrong RMSE can over- or under-smooth. |
| Median window | Broader spike detection. | More local and conservative. | Large windows can fill narrow drains. |
| Spike tolerance | Fewer cells corrected. | More cells corrected. | Too low alters real terrain. |
| Median max change | Bigger median corrections allowed. | Corrections capped more tightly. | Too high can fill/cut features. |
| Minimum valid neighbours | More conservative near gaps. | More cells can be processed. | Too high may skip useful corrections. |
| Bilateral window | More neighbours considered. | More local smoothing. | Large windows can soften features. |
| Spatial sigma | Farther cells have more influence. | Only close cells influence strongly. | Too high can broaden smoothing. |
| Range sigma | Larger height differences are smoothed together. | Height breaks are preserved more strongly. | Too high can smooth across edges. |
| Noise threshold | Small changes are ignored. | Fine changes are applied. | Too high leaves noise; too low may make unnecessary adjustments. |
| Bilateral max change | Stronger per-cell smoothing allowed. | Output stays closer to input. | Too high can alter important features. |
| Gaussian radius/sigma/passes | Broader, softer surface. | Less broad averaging. | Can blur/round civil features. |

# 12. Scenario Recommendations

| Scenario | Starting approach | Notes |
| --- | --- | --- |
| 1 m LiDAR DEM, vertical RMSE about 0.035 m | Start with Light. Review Medium only if the output remains too noisy. | Light gives Median tolerance about 0.140 m and Bilateral range sigma about 0.053 m. |
| Drainage channels / stream beds | Use Light. Keep Bilateral window 3 and low range sigma. Use low max-change caps. | Check long sections along the flow path. Avoid Strong and avoid Gaussian. |
| Road corridors / kerbs | Use Light or very conservative manual values. | Lower range sigma preserves kerb breaks better. |
| Broad open terrain | Use Medium. Consider Strong only after visual checks. | Broad terrain is less sensitive to small edge damage. |
| Vegetation/spike affected terrain | Use Median ON with tolerance high enough to catch only true spikes. | Do not set spike tolerance too low; it can treat real features as spikes. |
| Presentation/general surface smoothing | Use Bilateral Medium first. Gaussian may be used only where broad averaging is acceptable. | Document that output elevations are changed. |

# 13. Runtime Guidance

Runtime depends mainly on grid size, Median window size, Bilateral window size, Gaussian radius, and Gaussian passes.

A 5×5 window checks many more cells than a 3×3 window.

Larger Bilateral windows are slower because both spatial and range weights are evaluated.

Gaussian passes increase runtime approximately linearly.

For large grids, test on a copy or a smaller area before processing the full surface.

# 14. Output Report

When processing finishes, the macro prints a report to the 12d Output Window.

| Report item | Meaning |
| --- | --- |
| Input grid / output grid / output model | Confirms what was processed and where the result was written. |
| Input data size / grid dimensions / spacing | Helps assess runtime and whether resolution values are sensible. |
| Loaded input grid height cells | Valid grid cells read from the input. |
| Written output grid height cells | Valid cells written to the output. |
| Skipped/undefined cells | Cells without valid heights. |
| Median altered cells | Cells changed by the Median stage. |
| Median max-change clipped cells | Median corrections capped by Median maximum vertical change. |
| Bilateral altered cells | Cells changed by the Bilateral stage. |
| Bilateral max-change clipped cells | Bilateral corrections capped by Bilateral maximum vertical change. |
| Gaussian altered cells | Cells changed by the Gaussian stage. |
| Maximum lowering / maximum raising | Largest negative and positive output changes. |
| Mean vertical change | Overall vertical bias. Large positive or negative mean should be reviewed. |
| Standard deviation of change | Spread of output changes. |
| Processing time | Total processing time in seconds. |

Difference definition used by the macro:

ΔZ = output Z − input Z

Negative ΔZ means the output is lower than the input.

Positive ΔZ means the output is higher than the input.

# 15. Visual Checks

Always compare the original and smoothed outputs. Contours and section profiles are often more useful than plan view alone.

Figure 2: Example TIN contours generated from the original/pre-processing String_Grid.

Figure 3: Example TIN contours generated from a smoothed String_Grid.

Figure 4: Side-by-side contour comparison. The smoothed result may improve readability, but critical terrain breaks must still be verified.

## Suggested checks

Cut sections across and along stream beds, drains, road crowns, kerbs, retaining walls, steep banks and batters.

Review areas with large maximum raising or lowering in the report.

Check for channel filling, kerb rounding, batter flattening, and artificial sinks or humps.

Keep the original grid available until the smoothed grid has been accepted.

# 16. Companion Spreadsheet

The supplied decision-support spreadsheet can be used to calculate and document starting parameters from grid resolution and vertical RMSE. It is useful for project records and for comparing Light, Medium, and Strong parameter sets before applying them in the macro.

The spreadsheet and the macro presets should be treated as starting points only. Final settings must be selected using site context, cross-section checks, and engineering judgement.

# 17. Quick Starting Point

| Item | Suggested starting value |
| --- | --- |
| String_grid resolution | 1.000 m |
| String_grid vertical RMSE | 0.035 m |
| Smoothing preset | Light |
| Median | ON; window 3; spike tolerance about 0.140; max change about 0.053; min neighbours 5 |
| Bilateral | ON; window 3; spatial sigma 1.000; range sigma about 0.053; noise threshold 0; max change about 0.035 |
| Gaussian | OFF |

Increase smoothing gradually only after the original and smoothed TINs have been compared.